# Day 11 – Registers, Timers and Interrupts

## 1. Prescaler (PSC)

• Controls how fast the timer counts.
• Divides the incoming clock before it reaches the timer.
• A larger PSC value makes the timer count more slowly.
• Used together with ARR to generate accurate timing intervals.

## 2. Auto Reload Register (ARR)

• Sets the maximum count value of the timer.
• When the counter reaches ARR, an Update Event occurs and the timer resets to 0.
• The Update Event can trigger an interrupt, DMA transfer, or another timer event.

## 3. NVIC (Nested Vectored Interrupt Controller)

• Receives interrupt requests from peripherals.
• If the interrupt is enabled, it notifies the CPU.
• The CPU saves its current state on the stack, executes the Interrupt Service Routine (ISR), and then resumes the original program.

## 4. volatile Keyword

• Variables shared between the main program and an ISR should be declared as volatile.
• This prevents the compiler from optimizing the variable and ensures its latest value is always read from memory.

## Summary

• PSC → Controls timer speed.
• ARR → Determines when the timer overflows.
• NVIC → Handles pending interrupts and directs the CPU to the ISR.
• Shared variable keyword → volatile.