# Day 10 – GPIO, Clocks and Memory Map

## 1. Memory Map

The STM32 has a 32-bit (4 GB) address space. Different parts of the chip occupy fixed address ranges.

• 0x08000000 → Flash (program memory)
• 0x20000000 → SRAM (variables, stack, heap)
• 0x40000000 → APB peripherals (RCC, USART, I2C, Timers)
• 0x50000000 → AHB peripherals (GPIO)

The main idea is that peripherals are memory-mapped, so accessing a register is the same as accessing a memory address.

## 2. GPIO Register Map (GPIOA)

| Offset | Register | Purpose |
| --- | --- | --- |
| 0x00 | MODER | Selects pin mode (Input/Output/AF/Analog) |
| 0x04 | OTYPER | Push-pull / Open-drain |
| 0x08 | OSPEEDR | Output speed |
| 0x0C | PUPDR | Pull-up / Pull-down |
| 0x10 | IDR | Read pin state |
| 0x14 | ODR | Write output state |
| 0x18 | BSRR | Set/Reset pins safely |

GPIOA base address: 0x50000000
GPIOA->ODR address = 0x50000014

## 3. RCC (Clock Control)

Before using any peripheral, its clock must be enabled through RCC. If the clock is disabled, the peripheral ignores register writes.

Common registers:
• IOPENR – GPIO clocks
• APBENR1 – I2C, USART2, Timers
• APBENR2 – USART1, SPI1, ADC

Example:
RCC->IOPENR |= (1 << 0);   // Enable GPIOA clock

## 4. Peripheral Address Ranges

| Peripheral | Base Address |
| --- | --- |
| GPIOA | 0x50000000 |
| RCC | 0x40021000 |
| USART1 | 0x40013800 |
| I2C1 | 0x40005400 |

## Key Takeaways

• GPIO is controlled through memory-mapped registers.
• Each GPIO pin uses 2 bits in MODER to select its mode.
• RCC clock must be enabled before using any peripheral.
• Pointers and structs make accessing registers easier in C.