# Day 12 Deliverable – UART Implementation Blueprint

## Chosen Protocol: UART

UART Frame (8N1)
Idle → Start(0) → D0 D1 D2 D3 D4 D5 D6 D7 → Stop(1)

## Initialization Sequence

Enable USART1 clock in RCC

Configure BRR (baud-rate divider)

Configure CR1 (UE, TE, RE, M0, PCE)

Enable USART

Wait for TXE

Write data to TDR

Receiver reads data from RDR

## Notes

8N1 = 8 data bits, No parity, 1 stop bit.

BRR stores the baud-rate divider calculated from Peripheral Clock / Baud Rate.

If the peripheral clock changes, BRR must be recalculated.

Use |= to set required bits without changing other bits.

TXE: ready to transmit, RXNE: data received, TC: transmission complete.